Ctrl-Alt-Del Inc. Project 1

Dennis Yanovski

November 13, 2024

Final Project

| Course Title | Introduction to Internetworking |
| --- | --- |
| Project Code | INN8 Project 1 |
| Project Title | Ctrl-Alt-Del Inc. |

# Introduction to Internetworking

Project 1: Ctrl-Alt-Del Inc.

## INTRODUCTION

You have accepted a contract to participate in the design of the network infrastructure of a company called Ctrl-Alt-Del Inc. whose operations are located in two offices at the extreme ends of Canada. The North Office is in Iqaliut, Nunavut, while the South Office is found in Pelee, Ontario. Your job is to develop IP addressing schemes at both locations as well as to assist in selecting routing protocols for the two sites.

## OBJECTIVES

The objectives of this project are:

- Design a valid IP addressing scheme for a given LAN

- Perform IP addressing and subnetting calculations

- Choose a routing protocol to meet specified requirements

## MATERIALS REQUIRED

Pen and paper (and possibly a calculator) are all the materials required to complete this project. No hardware or software is needed.

## PROJECT SPECIFICATIONS

The diagram below shows a general view of Ctrl-Alt-Del Inc.’s network with the two sites in Iqaluit and Pelee.

North

Office

Iqaluit, NU

South Office

Pelee, ON

11

subnets

8

subnets

Instructions

- Developing an IP addressing scheme for the South Office

The South Office site consists of 8 network subnets with the number of hosts as indicated in the table below.

| Subnet | Number of Hosts |
| --- | --- |
| A | 14 |
| B | 10 |
| C | 11 |
| D | 12 |
| E | 10 |
| F | 4 |
| G | 3 |
| H | 4 |

A decision has already been made to use private addressing on the internal LAN of the South Office and your boss wants you to use a single class C address and implement subnetting to accommodate all 8 subnets.

  - Choose an appropriate valid private IP address for the LAN.

For a Class C address, we have the option of choosing within the range 192.168.0.0 to 192.168.255.255.

Chosen IP: 192.168.1.0

  - Assigning a subnet mask for the LAN

    - What will the subnet mask have to be to allow enough subnet address space for all the 8 subnets and as much address space for host addresses as possible (Note: Assume zero-subnets cannot be used)

Since there are 8 subnets, we need at least 8 subnet addresses. A Class C address has a default subnet mask of 255.255.255.0 (24 bits for the network), which gives you only 1 subnet. We can extend the mask to create more subnets by borrowing bits from the host portion.

Since we need at least 8 subnets, 3 bits are required (since 2 to the power of 3 = 8).

Subnet Mask: The new subnet mask will be 255.255.255.224, or /27 in CIDR notation.

Breakdown (1.2.1) : To create 8 subnets for the South Office while maximizing the address space for hosts, a subnet mask of 255.255.255.224 (/27) would work. Starting with a Class C address (default mask of 255.255.255.0), we would need to borrow 3 bits for subnetting to achieve 8 subnets. This /27 mask leaves 5 bits for host addresses, which allows each subnet to accommodate up to 30 hosts.

    - What is the maximum number of hosts each subnet will be able to accommodate?

With a /27 subnet mask, each subnet will have 5 bits for hosts, allowing for 2 to the power of 5 −2=30 hosts per subnet

  - How many valid subnets will this subnetting create? (Assume the zero-subnet cannot be used)

With a /27 mask and 3 bits for subnets, 8 valid subnets would be created (excluding the zero-subnet).

  - Write down the subnet numbers of these subnets

| Subnet | Subnet Number |
| --- | --- |
| A | 192.168.1.32 |
| B | 192.168.1.64 |
| C | 192.168.1.96 |
| D | 192.168.1.128 |
| E | 192.168.1.160 |
| F | 192.168.1.192 |
| G | 192.168.1.224 |
| H | 192.168.1.0 |

  - Determine the valid host address ranges and broadcast address for each subnet.

| Subnet | Range of valid host addresses | Broadcast address |
| --- | --- | --- |
| A | 192.168.1.1 - 192.168.1.30 | 192.168.1.31 |
| B | 192.168.1.33 - 192.168.1.62 | 192.168.1.63 |
| C | 192.168.1.65 - 192.168.1.94 | 192.168.1.95 |
| D | 192.168.1.97 - 192.168.1.126 | 192.168.1.127 |
| E | 192.168.1.129 - 192.168.1.158 | 192.168.1.159 |
| F | 192.168.1.161 - 192.168.1.190 | 192.168.1.191 |
| G | 192.168.1.193 - 192.168.1.222 | 192.168.1.223 |
| H | 192.168.1.225 - 192.168.1.254 | 192.168.1.255 |

- Developing an IP addressing scheme for the North Office

The North Office consists of a head office with 11 divisions each in their own subnet with the number of hosts as indicated in the table below:

| Subnet | Number of Hosts |
| --- | --- |
| A | 30 |
| B | 7 |
| C | 296 |
| D | 200 |
| E | 253 |
| F | 75 |
| G | 180 |
| H | 210 |
| I | 65 |
| J | 95 |
| K | 220 |

You are to use a single private class B address and implement subnetting. (Note:

Assume zero subnets are not to be used in this exercise)

  - Choose an appropriate valid private IP address for the LAN

For a private Class B IP address we have the option of choosing from 172.16.0.0 to 172.31.255.255.

Chosen IP: 172.16.0.0

  - Choose a subnet mask that will leave enough host address space to accommodate all the hosts on the subnets assuming that the largest amount of hosts will never exceed 320, but due to future expansion the number of subnets may increase. Write down that subnet mask and the number of hosts it will allow on each subnet.

Mask: 255.255.254.0 /23 MASK

Number of Hosts: 510 total possible hosts ( 2 to the power of 9 – 2 = 510)

  - How many valid subnets will this create?

With a /23 mask, 7 bits are available for subnets totaling 127 valid subnets.

(2 to the power of 7 – 1 = 127 usable subnets)

| Subnet | Subnet Address |
| --- | --- |
| A | 172.16.0.0 |

  - Write down the subnet addresses of these subnets

| B | 172.16.2.0 |
| --- | --- |
| C | 172.16.4.0 |
| D | 172.16.6.0 |
| E | 172.16.8.0 |
| F | 172.16.10.0 |
| G | 172.16.12.0 |
| H | 172.16.14.0 |
| I | 172.16.16.0 |
| J | 172.16.18.0 |
| K | 172.16.20.0 |

  - Determine the remaining information, such as the valid host address ranges and broadcast addresses. Include all the information in the table below.

| Subnet | Range of valid host addresses | Broadcast address |
| --- | --- | --- |
| A | 172.16.0.1 - 172.16.0.254 | 172.16.0.255 |
| B | 172.16.2.1 - 172.16.2.254 | 172.16.2.255 |
| C | 172.16.4.1 - 172.16.5.254 | 172.16.5.255 |
| D | 172.16.6.1 - 172.16.7.254 | 172.16.7.255 |
| E | 172.16.8.1 - 172.16.9.254 | 172.16.9.255 |
| F | 172.16.10.1 - 172.16.11.254 | 172.16.11.255 |
| G | 172.16.12.1 - 172.16.13.254 | 172.16.13.255 |
| H | 172.16.14.1 - 172.16.15.254 | 172.16.15.255 |
| I | 172.16.16.1 - 172.16.17.254 | 172.16.17.255 |
| J | 172.16.18.1 - 172.16.19.254 | 172.16.19.255 |
| K | 172.16.20.1 - 172.16.21.254 | 172.16.21.255 |

- Choosing routing protocols and WAN technologies

Your last step is to help design a routing strategy for the Ctrl-Alt-Del, Inc. network.

Select appropriate routing protocols for the LAN at each of the two company offices. Write down your selections below:

Routing protocol for the North Office LAN:

Best Option:  For larger networks with varied subnets, OSPF would be a good choice, as it provides efficient routing for multiple subnets and can handle future scalability.

Routing protocol for the South Office LAN :

Best Option:  For smaller networks with fewer requirements, RIP (routing information protocol), would be sufficient. However, if a protocol with better efficiency is needed along with scalability, OSPF could be used here as well for consistency.

## MARKING SCHEME

| Grading Criteria and Subunits | Grading |
| --- | --- |
| IP Addressing Scheme for South Office |  |
| 1.1. Choose appropriate private IP address for the LAN | 5 |
| 1.2. Assigning a subnet mask for the LAN | 10 |
| 1.2.1. Subnet mask allows for enough subnet address space | 5 |
| 1.2.2. Maximum number of hosts calculation per subnet | 5 |
| 1.3. Number of valid subnets created (with zero-subnet assumption) | 5 |
| 1.4. Listing subnet numbers for all subnets | 5 |
| 1.5. Valid host address ranges and broadcast addresses for each subnet | 10 |
| IP Addressing Scheme for North Office |  |
| 2.1. Choose appropriate private IP address for the LAN | 5 |
| 2.2. Subnet mask selection based on host and future subnet needs | 10 |
| 2.3. Number of valid subnets created | 5 |
| 2.4. Listing subnet addresses for all subnets | 5 |
| 2.5. Valid host address ranges and broadcast addresses for each subnet | 10 |
| Choosing Routing Protocols and WAN Technologies |  |
| 3.1. Selection of routing protocol for North Office | 10 |
| 3.2. Selection of routing protocol for South Office | 10 |
| Documentation and Presentation |  |
| Clear and organized documentation of IP addressing schemes | 10 |
| Professional presentation of subnetting calculations and routing protocols | 10 |
| Total | 100 |

WHAT TO SUBMIT

This document is the only item you need to submit to your instructor.

## RUBRIC

| Grading Criteria | Insufficient | Acceptable | Good | Excellent |
| --- | --- | --- | --- | --- |
| IP Addressing Scheme for South Office |  |  |  |  |
| 1.1. Choose appropriate private IP address for the LAN | Incorrect or nonprivate IP address chosen. | Private IP chosen but does not fully align with LAN requirements. | Appropriate private IP address chosen for LAN but minor inefficiencies exist. | Optimal private IP address chosen, fully suitable for LAN requirements. |
| 1.2. Assigning a subnet mask for the LAN | No subnet mask provided or incorrect mask. | Subnet mask provided but inefficient for LAN needs. | Correct subnet mask with sufficient space, but small errors in logic. | Correct and efficient subnet mask with ample address space for the LAN. |
| 1.2.1. Subnet mask allows for enough subnet address space | No consideration for subnet address space. | Subnet mask allows for basic address space, but not future growth. | Subnet mask allows for sufficient address space and some future growth. | Subnet mask allows for optimal address space with consideration for future expansion. |
| 1.2.2. Maximum number of hosts calculation per subnet | Incorrect or missing host calculations. | Calculations made but contain minor errors. | Maximum number of hosts calculated correctly with minor issues in precision. | Accurate calculation of the maximum number of hosts per subnet with clear rationale. |
| 1.3. Number of valid subnets created | No valid subnets created or errors in subnetting logic. | Basic number of valid subnets created, but logic is partially flawed. | Correct number of valid subnets created with minor errors in listing. | Correct and optimized number of valid subnets created using the zero-subnet assumption. |
| 1.4. Listing subnet numbers for all subnets | Missing or incorrect listing of subnet numbers. | Partial or incomplete listing of subnet numbers. | Complete list of subnet numbers with some inconsistencies. | Accurate and complete listing of subnet numbers for all subnets. |
| 1.5. Valid host address ranges and broadcast addresses | Missing or incorrect host ranges and broadcast addresses. | Host address ranges and broadcast addresses listed, but contain errors. | Mostly correct ranges for valid hosts and broadcast addresses, with some inaccuracies. | Fully accurate host address ranges and broadcast addresses listed for all subnets. |
| IP Addressing Scheme for North Office |  |  |  |  |
| 2.1. Choose appropriate private IP address for the LAN | Incorrect or nonprivate IP address chosen. | Private IP chosen but does not fully align with LAN requirements. | Appropriate private IP address chosen for LAN but minor inefficiencies exist. | Optimal private IP address chosen, fully suitable for LAN requirements. |
| 2.2. Subnet mask selection based on host and future needs | Subnet mask does not meet current or future needs. | Subnet mask meets current needs but lacks consideration for future growth. | Subnet mask chosen with some provision for future host and subnet needs. | Subnet mask allows for optimal use of host and subnet address space with foresight for |
| Grading Criteria | Insufficient | Acceptable | Good | Excellent |
|  |  |  |  | future needs. |
| 2.3. Number of valid subnets created | No valid subnets created or errors in subnetting logic. | Basic number of valid subnets created, but logic is partially flawed. | Correct number of valid subnets created with minor errors in listing. | Correct and optimized number of valid subnets created. |
| 2.4. Listing subnet addresses for all subnets | Missing or incorrect listing of subnet addresses. | Partial or incomplete listing of subnet addresses. | Complete list of subnet addresses with some inconsistencies. | Accurate and complete listing of subnet addresses for all subnets. |
| 2.5. Valid host address ranges and broadcast addresses | Missing or incorrect host ranges and broadcast addresses. | Host address ranges and broadcast addresses listed, but contain errors. | Mostly correct ranges for valid hosts and broadcast addresses, with some inaccuracies. | Fully accurate host address ranges and broadcast addresses listed for all subnets. |
| Choosing Routing Protocols and WAN Technologies |  |  |  |  |
| 3.1. Selection of routing protocol for North Office | No routing protocol selected or incorrect protocol chosen. | Basic routing protocol selected but inefficient for office needs. | Suitable routing protocol selected with minor errors in logic or efficiency. | Optimal routing protocol selected based on office requirements and future needs. |
| 3.2. Selection of routing protocol for South Office | No routing protocol selected or incorrect protocol chosen. | Basic routing protocol selected but inefficient for office needs. | Suitable routing protocol selected with minor errors in logic or efficiency. | Optimal routing protocol selected based on office requirements and future needs. |
| Documentation and Presentation |  |  |  |  |
| Clear and organized documentation of IP addressing schemes | Documentation is unclear or disorganized, lacking key details. | Documentation is somewhat organized but missing some key details or clarity. | Clear documentation with a few minor areas needing more detail. | Highly clear, organized, and professional documentation of all IP addressing schemes. |
| Professional presentation of subnetting and routing | Presentation lacks professionalism, with errors in calculations and routing protocols. | Presentation includes basic calculations and protocols but lacks full clarity or professionalism. | Professional presentation with clear calculations and protocols but could be more polished. | Highly professional presentation with accurate calculations and routing protocols, well-structured and visually clear. |

\